User:

- bore_optimizer.py

please evaluaste these scripts ,they are from the backend of my project to simualte and design 3d printed instruments

Kimi:

Here is a detailed evaluation of bore_optimizer.py, organized by severity.

🔴 Critical Bugs

1. Optimized radii are discarded in the final result In the else branch (n_bore_cp > 0), you optimize length + radii, but when computing final metrics you call:

temp_net = self._make_network(best_length)  # Missing radii!

Fix: Pass the optimized radii through:

best_radii = result.x[1:] if self.n_bore_cp > 0 else None
temp_net = self._make_network(best_length, best_radii)

And include them in the returned parameters.

2. _make_network ignores intermediate control-point radii

net.segments[0] = Segment(
    length=bore_length,
    radius_in=bore_radii[0],
    radius_out=bore_radii[-1],
)

If n_bore_cp = 5, you pass 5 radii to the objective but only use radii[0] and radii[-1]. The optimizer is therefore optimizing over dummy variables.

Fix: Either interpolate a profile from the control points, or build a multi-segment bore:

# Example: linear interpolation between control points
cp_positions = np.linspace(0, bore_length, len(bore_radii))
# ... create multiple Segment instances ...

3. Segment is imported twice Once inside _make_network and once at module level. The inner import shadows the outer one and is unnecessary.

🟠 Major Issues

4. Missing Phase 2 (L-BFGS-B) Your docstring and class doc both claim a two-phase approach (DE global → L-BFGS-B local), but only DE is implemented. For instrument design, this matters: DE gets you near the basin, but L-BFGS-B sharpens the cents precision.

5. boundary_bell.position = 0.0 is suspicious Setting the bell to position 0 (the input/mouthpiece end?) after changing bore length looks like a bug. If your coordinate system places the bell at x = L, this should probably be:

net.boundary_bell.position = bore_length

6. Success threshold is too loose

success=best_cost < 50.0

50 cents = half a semitone. For a 3D-printed instrument, you likely want < 5–10 cents absolute RMS. Consider making this configurable.

7. evaluate swallows all exceptions

except Exception:
    return 1e10

This will silently mask solver failures, network construction bugs, or numerical instabilities. At minimum, log the exception type/message when verbose=True.

🟡 Design & Performance

8. Deep-copying the network on every evaluation

net = copy.deepcopy(self.network)

With DE (maxiter=50, default population 15×dim), this runs ~750+ times. If AcousticNetwork is heavy, this dominates runtime. Consider: - A lightweight network.clone_bore_only() method - Or mutating a template in-place with a context manager/restore

9. Hole position optimization is documented but not implemented The docstring says “Hole positions (optional)” but there are no parameters or bounds for hole geometry.

10. n_bore_cp bounds are hard-coded

bounds.append((5.0, 25.0))  # Always 5–25 mm radii

This assumes a clarinet/soprano scale. For a bass instrument or recorder, this is wrong. Make it a constructor argument.

🟢 Good Practices (Keep These)

- Absolute RMS as primary metric — correct choice per your references. Median-corrected RMS is a useful secondary diagnostic for “evenness” across the scale.

- Cents space — optimizing in log-frequency (cents) rather than Hz is the right domain for musical intonation.

- Type hints — clean and helpful.

- Separation of concerns — evaluate() is pure-ish (modulo the counter), optimize() handles the strategy.

📋 Recommended Refactor Priority

| Priority | Task |
| --- | --- |
| P0 | Fix _make_network to use all control-point radii |
| P0 | Fix final metrics to use the optimized radii |
| P1 | Add L-BFGS-B refinement phase after DE |
| P1 | Verify bell position logic |
| P2 | Replace deepcopy with a lighter bore-mutation method |
| P2 | Make success threshold and radius bounds configurable |
| P3 | Add hole-position bounds if you plan to optimize toneholes |
| P3 | Add logging in evaluate exception handler |

Would you like me to review the AcousticNetwork / Segment classes or the TMMSolver next? The interaction between _make_network and those classes is where most of the bugs originate.

User:

- base.py

Yes, and these as well, please.

Kimi:

Here is the evaluation of base.py.

🟡 Minor Issues

1. Unused imports

import numpy as np
from typing import List, Optional

np, List, and Optional are imported but never referenced in this file. Remove them to keep the module clean.

2. No __all__ export list Without __all__, from base import * pulls in ABC, abstractmethod, dataclass, etc. Add:

__all__ = ["OptimizationResult", "Optimizer"]

🟠 Architectural Observations

3. The ABC is very minimal — consider standardizing optimizer metadata BoreOptimizer manually tracks _n_evaluations, bore_length_bounds, and n_bore_cp, but the base class provides no contract for introspection. If you add more optimizers (e.g., a hole-position optimizer or a multi-objective NSGA-II variant), you will likely want:

class Optimizer(ABC):
    @property
    @abstractmethod
    def parameter_bounds(self) -> List[Tuple[float, float]]:
        """Return bounds as a list of (low, high) tuples."""
        ...

    @property
    @abstractmethod
    def parameter_names(self) -> List[str]:
        """Human-readable names for each parameter dimension."""
        ...

This makes UI integration, logging, and cross-optimizer benchmarking much easier.

4. evaluate takes Dict[str, Any] — type-safe alternative The dict interface is flexible, but it pushes parameter validation to runtime. Consider a NamedTuple or TypedDict for each optimizer subclass, or at least add a validation hook:

@abstractmethod
def validate_parameters(self, parameters: Dict[str, Any]) -> None:
    """Raise ValueError if parameters are malformed."""
    ...

5. optimize has no callback / progress hook For a GUI or CLI that runs for minutes, you currently have no way to stream intermediate results. Consider:

def optimize(
    self,
    verbose: bool = False,
    callback: Optional[Callable[[int, float, Dict[str, Any]], None]] = None
) -> OptimizationResult:

This would let the frontend plot convergence live.

🟢 Good Practices (Keep These)

- @dataclass for OptimizationResult — clean, self-documenting, and extensible via metadata

- field(default_factory=dict) — correctly avoids the mutable-default-argument pitfall

- Metric semantics in docstrings — explicitly labeling rms_cents as PRIMARY (accuracy) and rms_cents_median as SECONDARY (evenness) prevents future contributors from accidentally swapping them

- ABC + abstract methods — enforces the interface without imposing implementation details

📋 Suggested Refactor Priority

| Priority | Task |
| --- | --- |
| P2 | Remove unused imports; add __all__ |
| P2 | Add parameter_bounds / parameter_names abstract properties |
| P3 | Add optional callback to optimize signature |
| P3 | Add validate_parameters hook |

Bottom Line

base.py is a solid, clean interface. The main risk is that it is too minimal: as your backend grows (multi-objective optimizers, hole optimizers, joint bore+hole optimizers), you will want richer introspection hooks. Adding parameter_bounds and a progress callback now will save you from refactoring every concrete optimizer later.

Do you want me to review the AcousticNetwork / Segment classes next? That is where the geometry bugs in bore_optimizer.py actually need to be fixed.

User:

- coordinates.py

Yes please, and these:

Kimi:

Here is the evaluation of coordinates.py.

🔴 Major Issues

1. Default bore_length=0.0 is a silent foot-gun

def openwind_to_internal(x_openwind: float, bore_length: float = 0.0) -> float:

If a caller forgets to pass bore_length, every position becomes negative (0.0 - x). Because this module is explicitly safety-critical (“No code outside this module should ever convert coordinates directly”), a missing length should fail loudly, not produce silently invalid positions.

Fix: Remove the default:

def openwind_to_internal(x_openwind: float, bore_length: float) -> float:

2. No input validation A “safety” module that accepts any float without bounds checking is not actually safe. Someone can pass x = 1500 for a 500 mm bore and get a valid-looking negative or out-of-bounds internal position.

Fix: Add validation:

if not (0.0 <= x <= bore_length):
    raise ValueError(f"Position {x} out of bounds [0, {bore_length}]")

🟠 Design Issues

3. The “no direct conversion” rule is unenforceable Your module docstring says: > “No code outside this module should ever convert coordinates directly.”

But because x is just a float, nothing stops another module from writing L - x. If you truly want to prevent mixing conventions at the type level, use typing.NewType:

from typing import NewType

InternalPos = NewType("InternalPos", float)
OpenWindPos = NewType("OpenWindPos", float)
ChalumierPos = NewType("ChalumierPos", float)

def openwind_to_internal(x: OpenWindPos, bore_length: float) -> InternalPos:
    ...

This makes it impossible to pass an OpenWindPos to a function expecting InternalPos without an explicit cast through this module.

4. Static methods force bore_length to be passed everywhere If you instantiate a transform for a specific instrument, you eliminate an entire class of “forgot to pass length” bugs:

class InstrumentCoords:
    def __init__(self, bore_length: float):
        self.L = bore_length
    
    def openwind_to_internal(self, x: float) -> float:
        ...

This pairs naturally with removing the dangerous default from issue #1.

🟡 Minor Issues

5. Missing type hints For a module whose entire job is type/domain safety, the lack of hints is notable.

6. Naming inconsistency - Module docstring: OpenWInD - Method names: openwind (lowercase) - Method docstrings: OpenWInD (mixed case)

Pick one and stick to it. I recommend OpenWind (camel case) for prose and openwind (snake case) for identifiers.

7. Missing __all__

__all__ = ["CoordinateTransform"]

🟢 Good Practices (Keep These)

- Explicit identity transforms — chalumier_to_internal being an identity function looks odd, but it is excellent API design. It makes the convention explicit at every call site and future-proofs you if Chalumier ever changes its origin.

- Centralized conversion — The architectural decision to funnel all coordinate math through one module is exactly right for a multi-solver backend (TMM, FEM, OpenWind export, etc.).

- Clear convention documentation — The module-level docstring stating 0 = bell, L = reed is essential. Keep this visible.

📋 Recommended Refactor Priority

| Priority | Task |
| --- | --- |
| P0 | Remove bore_length=0.0 default; make it required |
| P0 | Add bounds validation (0 <= x <= L) |
| P1 | Add type hints; consider NewType for coordinate flavors |
| P2 | Convert static methods to an instance class tied to a specific instrument length |
| P2 | Normalize OpenWind / openwind naming |
| P3 | Add __all__ |

Bottom Line

coordinates.py has the right architecture but not enough safety. The combination of a zero default and no bounds checking means this “safety module” can actually manufacture subtle coordinate bugs. Making bore_length required and adding NewType wrappers would turn it into a genuine correctness barrier.

Do you want me to review the AcousticNetwork / Segment classes next? That is where the geometry bugs in bore_optimizer.py (e.g., _make_network discarding radii and the suspicious bell position logic) need to be resolved.

User:

yes

Kimi:

Please upload the files containing AcousticNetwork and Segment — likely something like network.py or core/network.py based on the imports I saw in bore_optimizer.py.

Kimi:

This appears to be the same coordinates.py file I already reviewed. I’m still waiting for the files that define AcousticNetwork and Segment — those are the ones needed to diagnose the geometry bugs in bore_optimizer.py (the discarded radii and the suspicious bell position logic).

Could you upload network.py (or core/network.py) and segment.py?

User:

- instrument_library.py

Ca n’t find it yet, here are some more scripts, while I’m looking:

Kimi:

Here is the evaluation of instrument_library.py.

🔴 Critical Issues

1. Duplicate entries in LIBRARY Several instruments appear twice with slightly different metadata, which will cause confusion in the UI and duplicate search results:

- “Modern Chalumeau in C (Clarinet)” — appears once under Woodwind (source: Printables – Tom) and again under Verified Professional (source: Printables – Tom_1766913)

- “C Clarinet Remix (14mm Bore)” — appears under Woodwind and again under Verified Professional

- “Diplica”, “Sipsi”, “Zummara” — appear under Woodwind with empty download_url, then again under Verified Professional with the same empty URLs

Fix: Deduplicate. If you need both a “browse” view and a “verified” view, add a boolean field verified: bool = False rather than duplicating the entire record.

🟠 Major Issues

2. Inconsistent case handling in query functions

def get_by_family(family: str) -> list[InstrumentEntry]:
    return [e for e in LIBRARY if e.family.lower() == family.lower()]  # case-insensitive

def get_subcategories(family: str) -> list[str]:
    # ...
    if e.family == family and ...  # case-SENSITIVE — bug!

A query for get_subcategories("wind") returns nothing because "wind" != "Wind", but get_by_family("wind") works. Same issue in get_type_labels.

Fix: Standardize all filters to case-insensitive comparison, or normalize inputs.

3. save_novel_instrument has fragile path logic

out_dir = output_dir or str(
    Path(__file__).parent.parent.parent / "test_output" / "unconventional" / "novel_instruments"
)

This assumes the file lives exactly three levels deep from the project root. If you move this module, it silently writes to a different location or fails.

Fix: Make output_dir a required parameter, or read it from an environment variable / config file.

4. save_novel_instrument imports inside the function

import json
from pathlib import Path

These should be at module level. Importing inside a function on every call adds overhead and makes dependencies non-obvious.

🟡 Design & Data Quality

5. LIBRARY is a mutable global

LIBRARY: list[InstrumentEntry] = [...]

Any code can append, delete, or reorder this at runtime. Since this is essentially static reference data:

Fix: Use a tuple:

LIBRARY: tuple[InstrumentEntry, ...] = (...)

And return list(LIBRARY) from getters if mutation is needed downstream.

6. No validation on InstrumentEntry difficulty accepts any string, but your data only uses Beginner, Intermediate, Advanced, Expert. A typo like "Intermidiate" would go unnoticed.

Fix: Add a DifficultyLevel enum or validate in __post_init__.

7. range field terminology is inconsistent You use: Soprano, Alto, Tenor, Bass, Contra, Sub-Contra, Sub-contrabass (mixed case). This will break filtering.

Fix: Normalize to an enum:

class Range(str, Enum):
    SOPRANO = "Soprano"
    ALTO = "Alto"
    # ...

8. type_label is free-form but acts like a taxonomy Values like "Fipple Flute", "Vessel Flute", "Single Reed", "Membrane/Double Reed" are scattered. If the frontend builds filters from these, inconsistent spacing or punctuation will create duplicate categories.

9. Empty download_url on “verified” instruments Several entries under the Verified Professional section have download_url="" (e.g., Diplica, Sipsi, Zummara, Octo-Contra instruments). If the UI shows a “Download” button based on this field, it will be a broken link.

🟢 Good Practices (Keep These)

- @dataclass — clean, extensible schema

- field(default_factory=list) — correctly avoids mutable default pitfall for tags

- save_novel_instrument dual API — gracefully handles both the new positional result dict and the legacy keyword arguments

- safe_name sanitization — strips unsafe characters before writing files

- Rich metadata — image_url, audio_url, tags, description make this a genuinely useful content module, not just a bare index

📋 Recommended Refactor Priority

| Priority | Task |
| --- | --- |
| P0 | Remove duplicate entries; add verified: bool flag instead |
| P0 | Fix case sensitivity in get_subcategories and get_type_labels |
| P1 | Move json/Path imports to module level |
| P1 | Replace mutable LIBRARY list with a tuple |
| P2 | Add DifficultyLevel and InstrumentRange enums |
| P2 | Normalize type_label to a controlled vocabulary |
| P2 | Make output_dir in save_novel_instrument required or config-driven |
| P3 | Add a printable: bool field to distinguish STL-available vs reference-only instruments |

Bottom Line

instrument_library.py is well-structured reference data, but the duplicate entries and inconsistent case handling are real bugs that will surface in the frontend as duplicate cards and missing filter results. The save_novel_instrument function is useful but needs to stop guessing file paths based on __file__ depth.

Still waiting on AcousticNetwork / Segment when you find them!

User:

can you look at the repo on kooshikooo-lab/instrument-designer, to try to find the file names?

Kimi:

Now I have the key files from your repo. Here is the comprehensive evaluation of `network.py`, `tmm_solver.py`, and `tmm_acoustics.py`, including the root causes of the bugs I flagged in `bore_optimizer.py`.

backend/core/network.py

🔴 Critical Issues

1. boundary_bell.position default is wrong

boundary_bell: Boundary = field(default_factory=lambda: Boundary(
    type=BoundaryType.BELL, excitation=ExcitationType.NONE, position=0.0
))

The bell is at position 0.0 (the open end), but boundary_reed is also at 0.0. If the code ever uses boundary_reed.position to place the reed, both boundaries collide at the same coordinate. For a bore of length L, the reed should be at position=L.

Fix:

boundary_reed: Boundary = field(default_factory=lambda: Boundary(
    type=BoundaryType.REED, excitation=ExcitationType.REED, position=0.0  # Actually, this should be L...
))

Wait — looking at your coordinate system: 0 = bell, L = reed. So boundary_bell.position = 0.0 is correct, but boundary_reed.position should default to total_length or be updated when segments change. Currently it is hard-coded to 0.0, which is the bell position.

Fix: Make boundary_reed.position a dynamic property or update it when segments are modified:

@property
def reed_position(self) -> float:
    return self.total_length

2. to_bore_profile() ignores n_points discretization Segment has n_points: int = 10 for FEM discretization, but to_bore_profile() only outputs segment endpoints. If a conical segment needs internal discretization for TMM accuracy, it is lost.

🟠 Major Issues

3. Fingering.port_states is a raw dict No type safety on the keys (should be int) or values (should be bool). A typo like "1" instead of 1 creates a silent missing-key bug.

Fix:

port_states: Dict[int, bool] = field(default_factory=dict)

4. get_port_states returns False for missing keys

return [fingering.port_states.get(i, False) for i in range(len(self.ports))]

False means “closed”. If a fingering omits a port by accident, it defaults to closed — a safe but potentially wrong assumption. Consider raising KeyError for incomplete fingerings in debug mode.

5. No validation that port positions are within bore length A Port at position=2000 on a 500 mm bore is silently accepted.

🟢 Good Practices (Keep These)

- Instrument-agnostic design — The solver “does NOT know whether it’s solving a clarinet or a trumpet.” This is excellent architecture.

- to_bore_profile() and to_hole_data() — Clean bridge to TMM. The separation of geometry extraction from physics is exactly right.

- Explicit coordinate convention in docstring — 0 = bell, L = reed prevents an entire class of bugs.

backend/solvers/tmm_solver.py

🔴 Critical Issues

6. from_network assumes register vent is at index 0

# Register vent is at the first port position (closest to reed)
has_register = any(p.is_register_vent for p in network.ports)
if has_register:
    reg_state = "open" if n_register >= 2 else "closed"
    adjusted_fingerings = [[reg_state] + list(fs) for fs in fingering_sets]

This prepends the register vent state to the fingering list, assuming the register vent is the first port. But _prepare_phase in tmm_acoustics.py sorts events by position. If the register vent is not at position 0 (closest to bell), the TMM will apply the wrong state to the wrong hole.

Fix: Ensure register vents are sorted to the correct position in AcousticNetwork.ports, or pass states mapped by position rather than index.

7. from_network computes outer_diameter incorrectly

outer_diameter = max(radii) * 2.5  # rough outer diameter estimate

This is a magic number. For a clarinet with 15 mm bore, this claims 37.5 mm outer diameter — plausible. For a bass clarinet with 30 mm bore, it claims 75 mm — too thick. The end flange correction depends on this.

Fix: Either accept outer_diameter as a network property, or compute it from a wall-thickness parameter.

🟠 Major Issues

8. compute_frequencies ignores the target_wavelengths parameter name The docstring says target_wavelengths: target wavelengths to search near (mm), but the variable name in tmm_acoustics.py is wavelength_near. The parameter is passed correctly, but the naming mismatch is confusing.

9. Loss model integration is incomplete _apply_bore_losses and _apply_hole_losses compute loss arrays but are never called by compute_frequencies. The loss_model is passed to TMMInstrument but the solver’s from_network default is NoLoss(). Losses are effectively disabled in the optimization pipeline.

🟢 Good Practices (Keep These)

- Clean bridging — from_network is a well-contained adapter between your abstract model and chalumier’s TMM.

- Automatic register vent handling — Hiding the register vent state from the caller simplifies fingering charts.

backend/tmm_acoustics.py

🔴 Critical Issues

10. Profile.at() has an interpolation bug

t = (location - self.pos[lo]) / (self.pos[hi] - self.pos[lo])
return (1.0 - t) * self.high[lo] + t * self.low[hi]

It interpolates between self.high[lo] and self.low[hi]. For a cylindrical segment where high[lo] == low[hi], this works. But for a conical segment, high[lo] is the diameter at the top of segment lo, and self.low[hi] is the diameter at the bottom of segment hi. If lo and hi are the same segment, this interpolates between the top and bottom of that segment — but if they are adjacent segments with a step, it interpolates across the discontinuity using mismatched endpoints.

Fix: The interpolation should use self.high[lo] (top of lower segment) and self.low[lo] (bottom of lower segment) when location is within segment lo. Or better, use np.interp with proper handling.

11. as_stepped has an off-by-one / missing-diameter bug

for i in range(len(self.pos) - 1):
    new_pos.append(self.pos[i])
    # ... compute steps ...
    if n_steps <= 1:
        continue  # SKIPS adding the endpoint!
new_pos.append(self.pos[-1])

When n_steps <= 1, it continues without adding the segment endpoint to new_pos. But the final new_pos.append(self.pos[-1]) only adds the very last point. If the second-to-last segment has n_steps <= 1, its endpoint (which is also the start of the last segment) is missing.

Fix: Always append the segment endpoints, or restructure to not skip.

12. wavelength_near has an infinite loop risk

for _ in range(max_steps):
    # Check for sign change at right end
    if scores[-2] >= 0.0 and scores[-1] < 0.0:
        return evaluate(len(scores) - 2)
    # Extend left...
    # Extend right...
    step = step ** step_increase

If no sign change is found within max_steps, it falls through to “Return best guess”. But max_steps is 100 and step grows exponentially (step = step ** 1.05). If the initial guess is far from resonance, the probes can overflow to inf or underflow to 0.0, causing scorer to return NaN.

Fix: Add if not math.isfinite(new_w): break after computing new probes.

🟠 Major Issues

13. resonance_phase applies losses incorrectly

if self.loss_model is not None and seg_diameter > 0:
    radius = seg_diameter / 2.0
    loss_factor = self.loss_model.bore_loss(seg_length, radius, wavelength)
    if isinstance(loss_factor, complex):
        phase += -loss_factor.imag

The loss model’s bore_loss returns a complex factor, but the phase shift from losses should be arg(exp(-gamma * L)) = -Im(gamma * L). However, loss_factor is likely exp(-gamma * L), so arg(loss_factor) = atan2(Im(loss_factor), Re(loss_factor)), not simply -loss_factor.imag. For small losses, -imag approximates arg, but for larger losses it is inaccurate.

Fix: Use cmath.phase(loss_factor) instead of -loss_factor.imag.

14. compute_fingered_frequencies returns List[float], not np.ndarray The type hint in tmm_solver.py says -> np.ndarray, but tmm_acoustics.py returns a plain Python list. This is a type mismatch that can cause downstream code to fail if it expects NumPy array methods.

15. phase_cost uses math.sin(math.pi * deviation) ** 2 This is the Ernoult 2020 phase-based cost. It is smooth and well-behaved, but note that deviation is in units of half-wavelengths (since phase is in half-wavelength units in this code). If deviation = 0.5, sin(pi * 0.5)^2 = 1.0 — maximum cost. This is correct for a half-wavelength shift (one full period off), but the docstring should clarify that phase is in half-wavelengths, not radians.

🟡 Minor Issues

16. tanner and untanner domain confusion

def tanner(phase: float) -> float:
    return math.tan(phase * math.pi)
def untanner(x: float) -> float:
    return math.atan(x) / math.pi

These convert between “phase in half-wavelengths” and tangent domain. The factor of pi is correct (half-wavelength = pi radians), but the naming is non-standard. Consider phase_to_tan_domain / tan_domain_to_phase.

17. hole_length_correction docstring says “Returns 0 for closed holes” But the code returns 0.0 only at the top:

if closed:
    return 0.0

Then proceeds to compute outer_correction = 0.7 etc. This is correct (returns 0 early), but the docstring is slightly misleading because it doesn’t mention that open holes get a non-zero correction.

18. SPEED_OF_SOUND is hard-coded to 346100.0 mm/s This is ~20°C. For optimization accuracy, temperature should be a parameter. A 5°C change shifts pitch by ~3 cents.

🟢 Good Practices (Keep These)

- Faithful chalumier port — The code explicitly states it is ported from ResonanceMath.kt and Instrument.kt. Keeping the algorithm identical to a validated reference is the right choice.

- Metamaterial extensions — The MetamaterialSideBranch and MetamaterialSegment classes are well-designed extensions that don’t break the existing TMM pipeline.

- Phase-based resonance — Much faster and smoother than peak-matching. Correct choice for optimization.

- Action chain precomputation — _prepare_phase builds a flat list of actions, making resonance_phase a tight loop. Good for performance.

Root Cause Analysis: bore_optimizer.py Bugs

Now that I have seen the underlying classes, here is exactly why the bore_optimizer.py bugs happen:

| Bug in bore_optimizer.py | Root cause in network.py / tmm_acoustics.py |
| --- | --- |
| Optimized radii discarded | _make_network passes bore_radii to Segment(radius_in=..., radius_out=...), but Segment only stores two radii. The TMM tmm_instrument_from_radii expects an array of radii at evenly-spaced positions. The optimizer’s n_bore_cp control points are never interpolated into a profile. |
| boundary_bell.position = 0.0 suspicious | AcousticNetwork defaults boundary_bell.position = 0.0 and boundary_reed.position = 0.0. Setting the bell to 0 is correct (bell is at 0), but the reed should be at total_length. The optimizer’s line net.boundary_bell.position = 0.0 is redundant but harmless. The real bug is that boundary_reed is never updated to bore_length. |
| Deep-copy overhead | AcousticNetwork contains List[Segment], List[Port], etc. copy.deepcopy recursively copies everything. Since Segment is a @dataclass with no custom __deepcopy__, it is slow for large networks. |

📋 Recommended Fix Priority

| Priority | File | Fix |
| --- | --- | --- |
| P0 | network.py | Fix boundary_reed.position to default to total_length or be dynamic |
| P0 | tmm_acoustics.py | Fix Profile.at() interpolation bug (wrong endpoints) |
| P0 | tmm_acoustics.py | Fix as_stepped off-by-one skip |
| P1 | bore_optimizer.py | Interpolate n_bore_cp radii into a multi-segment bore profile |
| P1 | tmm_acoustics.py | Fix loss phase shift: use cmath.phase not -imag |
| P1 | tmm_solver.py | Fix register vent index assumption; map by position |
| P2 | tmm_acoustics.py | Add math.isfinite guard in wavelength_near |
| P2 | tmm_solver.py | Make outer_diameter a network property, not max(radii)*2.5 |
| P2 | network.py | Add port_states: Dict[int, bool] type hint |

Do you want me to also review tmm_acoustics_jax.py, pareto_optimizer.py, or cadquery_export.py? The JAX solver and the CAD export are likely the most complex parts of the backend.

User:

yes please

Kimi:

Here is the comprehensive evaluation of `tmm_acoustics_jax.py`, `pareto_optimizer.py`, and `cadquery_export.py`.

backend/tmm_acoustics_jax.py

🔴 Critical Issues

1. scan_step computes ALL branches unconditionally

def scan_step(phase, i):
    act = c_at[i]
    pp = phase + c_pl[i] / wavelength * 2.0          # pipe branch
    a0 = jnp.pi * bore_radii[c_ji0[i]] ** 2
    a1 = jnp.pi * bore_radii[c_ji1[i]] ** 2
    sh = jnp.floor(phase + 0.5)
    pj = _untanner(a1 / a0 * _tanner(phase - sh)) + sh  # junction2 branch
    # ... hole branch ...
    new_phase = jnp.where(act == 0, pp, jnp.where(act == 1, pj, ph))
    return new_phase, None

For every action, JAX evaluates the pipe advance, the junction2 tangent, AND the hole junction3 tangent, then masks the result with jnp.where. This is numerically dangerous: _tanner(x) explodes near x = 0.5 (where tan(π/2) diverges). For the “wrong” branch, phase - sh can be exactly 0.5, producing inf that poisons gradients even though it is masked out.

Fix: Use jax.lax.switch inside scan_step, or pre-partition the action chain into separate lax.scan calls per action type.

2. make_rms_cost uses mean-corrected RMS

phase_errs = jax.vmap(_phase_err)((tw, fs_padded))
corrected = phase_errs - jnp.mean(phase_errs)   # <-- BUG
return jnp.sqrt(jnp.mean(corrected ** 2))

This subtracts the mean error before computing RMS. The optimizer therefore converges to a relatively even scale, not an absolutely accurate one. A design that is 50 cents sharp on every note returns 0 cost. This directly contradicts your OptimizationResult docstring which labels rms_cents as “PRIMARY — accuracy”.

Fix: Remove the mean correction:

return jnp.sqrt(jnp.mean(phase_errs ** 2))

3. MAX_HOLES = 24 silently truncates large instruments

fs_padded = jnp.zeros((n, MAX_HOLES + 1))
for i, fs in enumerate(fingering_sets):
    fs_padded = fs_padded.at[i, :len(fs)].set(fs)

If an instrument has >24 holes, extra hole states are silently dropped. A contra-bass clarinet with 24 toneholes + 1 register vent would lose the last hole state, causing wrong fingerings and bogus intonation.

Fix: Add assert max(len(fs) for fs in fingering_sets) <= MAX_HOLES or make MAX_HOLES configurable.

4. make_phase_cost uses phase - reg instead of phase - round(phase)

phase = resonance_phase(wl, bore_radii, fs_p)
return phase - reg    # reg = n_register (e.g., 1 or 2)

If the target wavelength guess is off by more than ±0.5 half-wavelengths, phase wraps to a different integer and phase - reg becomes misleadingly small (e.g., phase = 2.1, reg = 1, error = 1.1, but actual nearest resonance is at round(2.1) = 2, error = 0.1).

Fix: Use phase - jnp.round(phase) for robustness, or verify target wavelengths are pre-validated.

🟠 Major Issues

5. _find_resonance_bisect has no early stopping

(wl, wr, sl, sr, _, _), _ = lax.scan(expand_step, init_carry, None, length=8)
(wl, wr, sl, sr), _ = lax.scan(bisect_step, (wl, wr, sl, sr), None, length=10)

Exactly 8 expand + 10 bisect steps are executed regardless of convergence. For a well-behaved resonance, 3 bisect steps might suffice; for a difficult one, 10 may not.

Fix: Use jax.lax.while_loop with a convergence tolerance, though this complicates vmap. Alternatively, increase length and accept the overhead.

6. multi_objective_cost_v2 hard-codes 7.25 mm radius

jnp.mean((bore_radii - 7.25) ** 2)

This assumes all instruments have ~14.5 mm bore diameter. Wrong for bass clarinets (~12 mm radius) or piccolos (~5 mm).

Fix: Pass target_radius as a parameter.

7. build_action_chain_v2 may receive JAX tracers

template_diameters = [r * 2.0 for r in bore_radii_template]

If called during optimization with a JAX tracer, iterating with for r in ... fails.

Fix: Explicitly convert: bore_radii_template = np.asarray(bore_radii_template).

🟢 Good Practices (Keep These)

- lax.scan for the action loop — Eliminates Python overhead, enables JIT + grad

- jax.vmap for batching — Clean parallel evaluation across fingerings

- Phase-based cost for differentiability — Correct choice vs root-finding for gradient-based optimization

- Constant capture pattern — Chain arrays as closure constants is the canonical JAX approach

backend/pareto_optimizer.py

🔴 Critical Issues

8. pareto_sweep fixes bore length at L_init

intonation, timbre = evaluate_bi_objective(
    radii, L_init, hp, hd, hl, ...   # length is FIXED
)

The design vector x contains radii, hole positions, and hole diameters — but not bore length. The Pareto sweep cannot discover that a longer bore might improve timbre at the cost of intonation, or vice versa.

Fix: Include bore_length in x and bounds.

9. run_pareto uses L_est instead of the actual instrument length

wl_min = c / max(targets)
L_est = wl_min / 2.0 * 1.2
# ...
intonation, timbre = evaluate_bi_objective(
    radii, L_est, hp, hd, hl, ...   # rough estimate!
)

For conical bores or instruments with register vents, L_est can be off by 10–20%. The NSGA-II optimizer searches hole positions and radii for a bore of the wrong length.

Fix: Use cfg["bore_length"] or include length in the design vector.

10. _hole_radiation_consistency uses a single nominal bore radius

ratios = np.array([(d / (2.0 * bore_radius)) ** 2 for d in hole_diameters])

On a conical bore (e.g., saxophone: 12 mm → 22 mm), a 6 mm hole near the bell has a very different radiation ratio than the same hole near the reed. Normalizing by a single bore_radius makes the consistency metric meaningless for conical instruments.

Fix: Pass local bore radii at each hole position.

🟠 Major Issues

11. Manual O(n²) Pareto extraction is redundant

for i in range(len(F)):
    dominated = False
    for j in range(len(F)):
        if i == j: continue
        if (F[j, 0] <= F[i, 0] and F[j, 1] <= F[i, 1] ...):

pymoo’s NSGA2 already returns a non-dominated front in res.F. This double-filtering wastes time and may accidentally discard valid solutions due to floating-point equality.

Fix: Use res.F directly, or call res.opt if available.

12. compute_intonation_cost uses single n_register for all notes

freqs = inst.compute_fingered_frequencies(tw, fingerings, n_register)

But compute_fingered_frequencies accepts List[int] for per-note registers. A clarinet crossing the break needs register 1 for low notes and register 2 for high notes.

Fix: Accept n_register as a list.

13. _build_fingerings assumes simple-system sequential fingerings

fingerings.append(["open"] * (k + 1) + ["closed"] * (n_holes - k - 1))

This produces only cumulative fingerings (each note opens one more hole). It cannot model cross-fingerings, register vents, or chromatic key systems.

Fix: Document this limitation clearly, or load fingerings from the instrument config.

🟢 Good Practices (Keep These)

- Weighted-sum sweep before NSGA-II — Fast frontier exploration before expensive population search

- Bore-geometry timbre proxy — Clever workaround for slow impedance-peak computation

- Clear cost separation — Intonation and timbre as independent scalars makes the Pareto front interpretable

backend/cadquery_export.py

🔴 Critical Issues

14. INSTRUMENTS dict duplicates instrument_library.py cadquery_export.py contains ~100+ instrument specs. Many entries (e.g., toms_chalumeau_C, c_clarinet_remix_14mm, membrane_clarinet_nicolas_bras) also exist in instrument_library.py with slightly different metadata. Updating hole positions in one file requires manual sync with the other.

Fix: Make instrument_library.py the single source of truth. Have cadquery_export.py import specs and convert them to CadQuery kwargs.

15. Conical bore hole cuts may not fully intersect the bore

solid = _cut_holes(solid, holes, wall_thickness, hole_depth, lambda pos: (
    bore_diameter / 2 if not isinstance(bore_diameter, (list, tuple))
    else (small_d + pos / bore_length * (large_d - small_d)) / 2
))

_cut_single_hole creates a horizontal cylinder at x = inner_radius * side. For a conical bore, the inner surface is sloped. A horizontal cylinder of length wall_thickness + hole_depth may: - Not fully reach the bore (if hole_depth is too small) - Over-cut into the opposite wall (if the bore narrows sharply)

Fix: Use generate_variable_bore_instrument for all conical instruments, or verify hole-cylinder intersection with the local bore normal.

16. generate_folded_bore_instrument assumes constant bore radius

inner_r = bore_diameter / 2.0
# ...
x = 2 * bend_radius_mm + inner_r * side   # side offset

For a conical folded bore (e.g., folded bass clarinet), inner_r changes along the length. The hole side offset is wrong everywhere except at the nominal diameter.

Fix: Pass inner_radius_at as a callable, like the other generators.

🟠 Major Issues

17. generate_variable_bore_instrument has no monotonicity check

offsets = [positions[i] - positions[i-1] if i > 0 else 0 for i in range(len(positions))]

If positions are not strictly increasing, CadQuery’s loft will fail or produce twisted geometry. No validation.

Fix: Add assert all(positions[i] < positions[i+1] for i in range(len(positions)-1)).

18. generate_by_name strips _meta

spec = {k: v for k, v in INSTRUMENTS[name].items() if k != "_meta"}

The metadata (display name, description, source URL) is discarded. If the frontend wants to show instrument info alongside the download, it must duplicate the lookup.

Fix: Return _meta in the result dict.

19. Default hole_depth = wall_thickness + 2 may be insufficient For thick-walled instruments (bass clarinet: 6 mm wall), the hole cylinder extends only 2 mm past the inner wall. If the hole axis is not perfectly normal to the bore surface, the cut may not fully open.

Fix: Use hole_depth = wall_thickness + 0.5 * local_bore_diameter as a safety margin.

🟢 Good Practices (Keep These)

- Three generator functions — Simple, profile-based, and folded — good separation of concerns

- Hole side alternation — side = 1 if i % 2 == 0 else -1 reduces overhangs for 3D printing

- Export timing — Useful for benchmarking STL/STEP generation performance

- Comprehensive instrument library — Impressive coverage of historical, standard, and 3D-printed designs

📋 Cross-Critical Summary

| Priority | File | Issue | Impact |
| --- | --- | --- | --- |
| P0 | tmm_acoustics_jax.py | make_rms_cost mean-corrects phase errors | JAX optimizer optimizes evenness, not absolute pitch. Your “<3 cents RMS” benchmark may be measuring relative tuning, not absolute accuracy. |
| P0 | tmm_acoustics_jax.py | scan_step evaluates all branches unconditionally | NaN/inf gradients from tangent singularities on masked branches. Can crash optimization or produce wrong updates. |
| P0 | pareto_optimizer.py | pareto_sweep and run_pareto fix bore length | Multi-objective search cannot trade length for timbre/intonation. May miss true Pareto front. |
| P0 | cadquery_export.py | INSTRUMENTS duplicates instrument_library.py | Data drift: updating one file breaks the other. Maintenance nightmare. |
| P1 | tmm_acoustics_jax.py | MAX_HOLES=24 truncates large instruments | Contra-bass clarinets lose hole states silently. |
| P1 | cadquery_export.py | Conical hole cuts use horizontal cylinder | May under-cut or over-cut on sloped bore walls. |
| P1 | pareto_optimizer.py | _hole_radiation_consistency uses single bore_radius | Timbre metric is wrong for conical instruments (saxophones, oboes). |

Bottom Line

The most dangerous bug is in tmm_acoustics_jax.py:make_rms_cost — the mean correction makes your JAX gradient-based optimizer converge to relative tuning, not absolute intonation. If your benchmark pipeline uses this cost function for the “refined” column, those 0.00c / 0.53c results represent scale evenness, not pitch accuracy. Verify whether benchmark_all.py uses make_rms_cost or make_rms_cost_with_search for the final numbers.

Do you want me to review jax_optimizer.py or benchmark_all.py next to trace exactly which cost function feeds your benchmark table?

User:

yes please

Kimi:

Here is the evaluation of `jax_optimizer.py` and `benchmark_all.py`.

backend/jax_optimizer.py

🔴 Critical Issues

1. eval_all hard-codes outer_diameter_mm=22.0

inst = tmm_instrument_from_radii(
    radii, bore_length, hp, hd, hl,
    outer_diameter_mm=22.0,  # HARDCODED
    closed_top=closed_top, cone_step=0.5,
)

This ignores cfg["outer_diameter"]. For a bass chalumeau with 28 mm outer diameter, the end flange correction is computed for a 22 mm body — a 6 mm error that shifts the fundamental by several cents. Since safe_eval_local calls this function, all four refinement stages in refine_sequential optimize against the wrong flange correction.

Fix: Pass outer_diameter as a parameter.

2. eval_all and jax_stage2_refine ignore custom fingerings Both functions build cumulative fingerings internally:

for k in range(n_holes):
    f = ['open'] * (k + 1) + ['closed'] * (n_holes - k - 1)
    fingering_sets.append(f)

If an instrument config specifies cross-fingerings (e.g., a chromatic key system), the optimizer silently replaces them with cumulative ones. The resulting geometry is wrong for the intended fingerings.

Fix: Accept cfg["fingerings"] and use it directly.

3. DE re-optimization is arbitrarily restricted to open-open instruments

if not closed_top and n_h > 0:
    # DE global re-optimization

Closed-top instruments (clarinets, chalumeaux) also have non-convex hole-interaction landscapes. Excluding them from global search is a missed opportunity.

Fix: Remove the not closed_top guard, or make it configurable.

🟠 Major Issues

4. jax_two_phase_optimize accepts but ignores parameters

def jax_two_phase_optimize(
    ...
    bore_bounds_range: tuple = (3.0, 18.0),
    hole_pos_bounds_range: tuple = (10.0, None),
    maxfevals: int = 5000,
    n_iters: int = 500,
    ...
):

None of these four parameters are referenced in the function body.

5. refine_sequential uses n_cp = 6 but benchmark_all.py sequential uses 8 Inconsistent control-point count between the sequential init and the refined path. The refined optimizer starts from a 6-CP profile but the benchmark sequential uses 8. This can cause discontinuities in the optimization landscape.

6. Stage 4 bounds may be inconsistent after Stage 1 length change

hole_hi[i] = (hp[i + 1] - GAP) if i < n_h - 1 else (L * 1.3 - 30.0)

hole_hi is computed before Stage 1 optimizes bore length. If Stage 1 increases L by 15%, the precomputed hole_hi may still be valid (it allows up to 130%), but the intermediate holes’ upper bounds (hp[i+1] - GAP) become stale because they depend on the old neighbor positions.

🟢 Good Practices (Keep These)

- 4-stage refinement pipeline — Length → radii → holes → simultaneous is a well-established strategy (Noreland 2013)

- safe_eval wrapper — Catches TMM failures and returns 1e10 instead of crashing the optimizer

- Absolute RMS in eval_all — No median correction, so the optimizer targets absolute pitch accuracy

- use_jax_bore flag — Clean toggle between finite-difference and autodiff paths

backend/benchmark_all.py

🔴 Critical Issues

7. sequential_refined has wrong tuple unpacking

def sequential_refined(cfg):
    rms, L, radii, hp, hd, hl, t_refine = refine_sequential(...)
    # But the caller unpacks as:
    rms, L, hp, hd, dt = result[0], result[1], result[2], result[3], result[-1]

result[2] is radii (6 bore radii), but it is assigned to hp (hole positions). result[3] is hp, but assigned to hd (hole diameters).

Impact: The benchmark output prints len(hp) as the hole count — but hp is actually radii, so it prints 6 holes for every instrument. And hd contains hole positions, not diameters.

Fix: Match the 7-element return signature:

rms, L, radii, hp, hd, hl, dt = result

8. Multiple instruments have outer_diameter < inner_diameter (physically impossible)

| Instrument | Inner Diameter | Outer Diameter | Problem |
| --- | --- | --- | --- |
| pvc_flute_D | 20.4 mm | 14.0 mm | Negative wall thickness |
| concert_flute_C | 19.0 mm | 16.0 mm | Negative wall thickness |
| alto_flute_G | 22.0 mm | 18.0 mm | Negative wall thickness |

The end flange correction end_flange_length_correction(outer, inner) receives a negative w = (outer - inner)/2, producing a nonsensical correction. This corrupts the fundamental frequency for these instruments.

Fix: Use realistic outer diameters (e.g., PVC 3/4” Sch40 = 26.7 mm OD; concert flute silver tube ≈ 19.8 mm OD).

9. bass_chalumeau_Bb has mismatched holes vs targets

"targets": [233.1, 261.6, 293.7, 311.1, 349.2, 392.0, 440.0, 466.2]  # 8 targets
"fingerings": [
    ["closed"] * 8,  # 8 holes?
    # ...
    ["open"]*7 + ["closed"],  # 8th hole closed for last note
]

8 targets should mean 7 holes (fundamental + 7 notes). But the fingerings have 8 elements. The sequential placement only creates 7 holes, so the 8th element is silently ignored. The last note (Bb3) is computed with all 7 holes open instead of the intended 7-open-1-closed fingering.

Fix: Remove the 8th hole state from fingerings, or add a 9th target.

🟠 Major Issues

10. sequential uses 8 control points, refine_sequential uses 6

# benchmark_all.py sequential()
bore_radii = np.full(8, cfg["bore_radius"])

# jax_optimizer.py sequential_placement()
n_cp = 6

The “Sequential” benchmark evaluates an 8-CP profile, but “Seq+Refined” starts from 6 CPs and optimizes those. The RMS improvement from sequential to refined is therefore partially from reducing dimensionality, not just optimization quality.

11. --dask / --no-dask arguments are misleading

parser.add_argument("--no-dask", action="store_true", help="Accepted for compatibility...")
parser.add_argument("--dask", action="store_true", help="Ignored (see --no-dask).")

The arguments are parsed but ignored. The README claims distributed computing via Dask, but the benchmark doesn’t use it.

12. n_register logic may be wrong for open-open instruments

n_reg = 1 if cfg["closed_top"] else 2

The comment says this is because open-open pipes have a “phantom 1st resonance” in the stepped-cylinder model. But for a smooth conical bore (saxophone, oboe), the true fundamental is the 1st resonance. Forcing n_register=2 skips the actual fundamental and targets the first overtone. This is a model limitation that should be documented prominently, not hidden in a ternary expression.

🟡 Minor Issues

13. diatonic_D_chalumeau fundamental mismatch Named “D chalumeau” but fundamental is C#4 (277.2 Hz). The scale is D major (C#-D-E-F#-G-A-B), but the instrument’s lowest note is the leading tone, not the tonic. Either rename or document this design choice.

14. recorder_C labeled “conical” but config is cylindrical

"desc": "Soprano recorder in C (open-open, conical)",
"bore_radius": 5.5,  # Cylindrical single radius

Recorders are tapered (conical). The benchmark uses a cylindrical approximation.

🟢 Good Practices (Keep These)

- Absolute RMS in eval_all — Explicitly documented as “absolute, not median-corrected” to prevent scale-evenness masking

- Per-note register support for chromatic flute — _n_registers list handles the break between registers correctly

- Instrument configs with fingerings — Self-contained benchmark definitions make reproduction easy

- Graceful failure handling — Try/except around each instrument prevents one failure from killing the whole suite

Benchmark Table Verdict

Your README claims:

| Instrument | Seq+Refined |
| --- | --- |
| Chalumeau C | 0.53c |
| Bass Chalumeau Bb | 0.00c |
| … | … |

Are these numbers trustworthy?

- Yes for absolute RMS — The benchmark path (use_jax_bore=False) uses eval_all with no median correction, so the numbers represent actual pitch accuracy.

- No for metadata — Due to bug #7, the printed hole counts and diameters in the benchmark output are wrong.

- No for flange-corrected instruments — Due to bug #8, the PVC flute, concert flute, and alto flute have physically impossible outer diameters. Their “Seq” RMS values are computed with wrong end corrections. The “Seq+Refined” values may have partially corrected this during hole optimization, but the bore length was still optimized with the wrong flange.

- No for bass chalumeau fingering — Due to bug #9, the last note uses a wrong fingering, so 0.00c is suspect.

The JAX path is NOT used in the benchmark — benchmark_all.py calls refine_sequential(use_jax_bore=False), so the mean-correction bug in tmm_acoustics_jax.py does not affect the published numbers. However, anyone calling jax_two_phase_optimize(use_jax_bore=True) will get mean-corrected (evenness) optimization instead of absolute accuracy.

📋 Recommended Fix Priority

| Priority | File | Fix |
| --- | --- | --- |
| P0 | benchmark_all.py | Fix sequential_refined unpacking (result[2] = radii, not hp) |
| P0 | jax_optimizer.py | Pass outer_diameter through to eval_all instead of hard-coding 22.0 |
| P0 | benchmark_all.py | Fix impossible outer diameters (PVC 26.7mm, flute ~19.8mm, alto ~22.8mm) |
| P1 | benchmark_all.py | Fix bass_chalumeau_Bb fingering (7 holes, not 8) |
| P1 | jax_optimizer.py | Accept and use cfg["fingerings"] instead of building cumulative ones |
| P2 | jax_optimizer.py | Enable DE re-optimization for closed-top instruments |
| P2 | benchmark_all.py | Remove or implement --dask arguments |
| P2 | benchmark_all.py / jax_optimizer.py | Standardize n_cp to 6 or 8 consistently |

Do you want me to also review two_phase_optimizer.py or chromatic_flute.py? The former may be an older version of the optimization pipeline, and the latter is critical for the chromatic instrument benchmark path.

User:

Yes. And please give some advise on how to prevent these kinds of bugs, I’m using A.I agents in the Opencode app, mostly the Big Pickle model, on two computers. I have a system to keep them in line, governance docs, but apparently it’s not good enough, or the models are not good enough, probably both.

Kimi:

Here is the evaluation of `two_phase_optimizer.py` and `chromatic_flute.py`, followed by governance advice for your AI agent workflow.

backend/two_phase_optimizer.py

🔴 Critical Issues

1. outer_diameter_mm=22.0 hard-coded in all three places Same bug as jax_optimizer.py. All calls to tmm_instrument_from_radii use 22 mm outer diameter regardless of the actual instrument. For a bass chalumeau with 28 mm OD, the end flange correction is wrong.

2. n_bore_ctrl = 6 is hard-coded inside Phase 2

n_bore_ctrl = 6   # Overwrites the function parameter!

The function signature accepts n_bore_ctrl but immediately overwrites it with 6. If a caller passes n_bore_ctrl=8, Phase 2 silently truncates the design vector.

3. bore_length is never optimized The two-phase pipeline optimizes radii, hole diameters, and hole positions — but not bore length. The bore_length parameter is passed through unchanged from initial guess to final result. This contradicts the bore_optimizer.py docstring which claims to optimize “Bore length, Bore profile (radii at control points)”.

4. closed_top=False hard-coded This means the optimizer can only model open-open instruments (flutes, recorders, saxophones). It cannot optimize closed-open instruments like clarinets or chalumeaux, despite those being in your instrument library.

5. Phase 1 uses n_register parameter, but Phase 2 uses detected_regs

# Phase 1
x1, cost1, t1 = phase1_de_search(
    ..., n_register=n_register, ...
)

# Phase 2
x2, cost2, t2 = phase2_lbfgsb_refine(
    ..., detected_regs=regs, ...
)

Phase 1 optimizes for a single register number, but Phase 2 uses per-note registers. If the initial n_register doesn’t match the detected registers, Phase 1 optimizes the wrong resonances. For a chromatic scale crossing the break, Phase 1 will be wrong for half the notes.

6. detect_registers catches bare except

for pr in range(1, max_reg + 1):
    try:
        wl = inst.find_resonance(...)
    except:
        continue

Any TMM failure (not just “resonance not found”) is silently skipped. If find_resonance has a bug for a particular register, that register is never considered and the optimizer may pick a suboptimal one.

🟠 Major Issues

7. phase_cost_with_offset in Phase 1 is register-agnostic The docstring correctly notes this: “register-agnostic evenness is wanted.” But for an instrument with per-note registers (like the chromatic flute), register-agnostic optimization means Phase 1 may optimize hole positions that help the wrong overtone. Phase 2 then starts from a poor basin.

8. note_to_freq parsing is fragile

if '*' in name:
    idx = name.index('*')
    mult = float(name[idx+1:])

A4*1.5 works, but A4*1.5hz fails because the hz check comes first. Also no validation for unknown note names.

9. x0 hole positions are sorted, but the sorting is done inside the cost function too

hp = sorted(x[n_bore_ctrl + n_holes:])

This means the optimizer sees a non-differentiable sorted() operation. L-BFGS-B expects smooth objectives. The sorting creates flat regions and discontinuities in the gradient.

Fix: Use ordered bounds instead: enforce hp[i] < hp[i+1] via bounds, not sorting.

🟢 Good Practices (Keep These)

- Frozen registers — Detecting registers once from initial geometry and freezing them is exactly right (Noreland 2013). Prevents register drift.

- peak_cost_nearest for Phase 2 — Correctly avoids the periodic minima of sin² phase costs. This is a sophisticated design choice.

- Explicit discussion reference — “Per desktop decision (Discussion #23)” shows you are tracking design rationale.

backend/chromatic_flute.py

🔴 Critical Issues

10. compute_frequencies uses 2*c/f guess for register 3

if reg == 2:
    target_wavelengths.append(self.C / f_target)
else:
    target_wavelengths.append(2.0 * self.C / f_target)

For register 3 (second overtone), the wavelength is 3 * c / (2 * f) for an open-open pipe, not 2 * c / f. The 2*c/f guess is actually the fundamental wavelength, which is 1.5× too long for the n=3 resonance. This may cause find_resonance to converge to the n=2 resonance instead.

Fix: Use n_register * c / f for the guess, or self.C / f_target * (n_register / 2) for open-open pipes.

11. _seq_fing opens holes from the wrong end

def _seq_fing(n_open: int) -> List[str]:
    fing = ['closed'] * 17
    for i in range(16, 16 - n_open, -1):
        fing[i] = 'open'
    return fing

This opens the highest-indexed holes (closest to the foot/bell). But on a flute, opening holes closer to the embouchure (mouthpiece) shortens the tube more. The Boehm system opens holes from the top down (embouchure toward foot), not bottom up.

Wait — let me re-read. The docstring says “sequential opening from the foot (bottom-up).” But on a real Boehm flute, the first hole opened for C#4 is the foot C# (lowest note, shortest tube). For D4, you also open the foot D. This is indeed bottom-up. But range(16, 16 - n_open, -1) opens indices 16, 15, 14… which are the last holes in the list (closest to the foot). That matches “bottom-up” physically. But the TMM walks from bell (position 0) to reed (position L). On your coordinate system, position 0 = bell, L = reed. The foot is at position 0 (bell end). So opening holes at high indices (close to reed/embouchure) actually shortens the tube more.

This is confusing because the docstring says “from the foot (bottom-up)” but the code opens holes from the embouchure end (top-down in terms of acoustic effect). For a flute, the embouchure is the “input” and the foot is the “output.” Opening holes near the embouchure has a larger pitch-raising effect.

I think the fingering logic is actually correct for acoustic effect (opening from embouchure toward foot raises pitch progressively), but the docstring description “from the foot (bottom-up)” is misleading. The holes are opened from the headjoint toward the foot, which is top-down in playing terms but bottom-up in your coordinate system (since position 0 = bell/foot).

12. make_simple_config builds wrong number of targets

targets = [f for _, f in self.CHROMATIC_NOTES[:n_p + 1]]

n_p = 7 (primary holes), so this takes 8 targets (indices 0-7): C4 through G4. But the simplified model has 7 holes, which should give 8 notes (C4 through C5 if diatonic, or C4 through G4 if chromatic with 7 holes). This is actually correct for 7 holes + closed = 8 states. But the docstring says “6-7 primary holes” and the function returns 8 targets regardless.

Wait — n_p = len(p_idx) = 7. n_p + 1 = 8. So 8 targets. With 7 holes, you get 8 fingerings (0-7 open). This is correct for a diatonic scale. But the primary holes include the C# vent (index 0), which is a register vent, not a tone hole. Using it as a primary hole for sequential fingering means the first “note” opens the register vent, which is wrong — register vents are not opened sequentially for scale notes.

Fix: Exclude the register vent from PRIMARY_HOLE_INDICES, or treat it separately.

🟠 Major Issues

13. BORE_POSITIONS starts at 0 (embouchure) but docstring says 0 = embouchure Wait, your coordinate system says 0 = bell, L = reed. But BORE_POSITIONS = [0, 50, 100, ...] with 0 at the embouchure. This is the OpenWind convention (0 = mouthpiece), not your internal convention (0 = bell).

Fix: Either reverse the positions ([655, 605, 555, ...]) or document that this model uses OpenWind coordinates and converts them.

14. HOLE_SPECS positions are from embouchure Same issue — positions 195-596 mm are from embouchure (mouthpiece), but your TMM expects positions from the bell (foot). For a 655 mm bore, a hole at 195 mm from embouchure is at 655 - 195 = 460 mm from the bell.

Fix: Convert positions: hole_pos_from_bell = SOUNDING_LENGTH - hole_pos_from_embouchure.

15. make_chromatic_config missing hole_positions, hole_diameters, hole_lengths

cfg = {
    "desc": "...",
    "closed_top": False,
    "targets": model.all_target_freqs,
    ...
    # No hole geometry!
}

eval_all in benchmark_all.py expects these fields. Without them, the benchmark will fail or use defaults.

🟢 Good Practices (Keep These)

- Per-note register lists — all_registers as a list is exactly right for instruments crossing the break

- Primary hole extraction — Reducing 17 holes to 7 for optimization is a necessary and well-thought simplification

- Validation output with octave purity check — Excellent diagnostic for TMM correctness

- Static data as class attributes — Clean separation of model parameters from instance state

📋 Summary Table

| Priority | File | Issue |
| --- | --- | --- |
| P0 | two_phase_optimizer.py | outer_diameter_mm=22.0 hard-coded |
| P0 | two_phase_optimizer.py | n_bore_ctrl=6 overwrites parameter |
| P0 | two_phase_optimizer.py | bore_length never optimized |
| P0 | two_phase_optimizer.py | closed_top=False hard-coded |
| P0 | chromatic_flute.py | Register 3 guess uses 2*c/f instead of 3*c/(2*f) |
| P0 | chromatic_flute.py | Hole positions use embouchure-origin, not bell-origin |
| P1 | two_phase_optimizer.py | Phase 1 uses single n_register vs Phase 2 per-note detected_regs |
| P1 | two_phase_optimizer.py | sorted() inside cost function breaks differentiability |
| P1 | chromatic_flute.py | Register vent included in primary holes for simple config |
| P2 | chromatic_flute.py | make_chromatic_config missing hole geometry fields |

Governance Advice: Preventing AI-Agent Bugs

You asked why these bugs persist despite governance docs. Here is the honest answer: your governance system is designed for human developers, not for LLMs. LLMs don’t “read” docs the way humans do — they pattern-match. Your current setup has three structural failures.

1. The “Big Pickle” Problem

Big Pickle (and most large LLMs) has a context window limit (~128k tokens) and a recency bias (stronger attention to the end of the prompt). If your governance doc is 5,000 words and you append it to every prompt, the model: - Remembers the style of the doc (“be careful with coordinates”) - Forgets the specific rule (“bore_length must be passed to eval_all”) by the time it reaches the code - Hallucinates “fixes” that look correct but violate constraints it can no longer see

What to do instead:

A. Use a “Constraint Checklist” pattern, not a governance essay Instead of: > “Always verify that outer_diameter is passed correctly and that coordinate systems are consistent…”

Use:

## BEFORE WRITING CODE, CHECK:
- [ ] `outer_diameter` passed to `tmm_instrument_from_radii`? (default 22.0 is WRONG)
- [ ] `closed_top` matches instrument family? (clarinet=True, flute=False)
- [ ] `bore_length` included in optimization bounds?
- [ ] `n_bore_ctrl` respected from caller, not hard-coded?

Force the model to output the checklist before code. This triggers explicit reasoning.

B. Use “Rule Files” as RAG, not prompt stuffing Store your constraints as a vector database (or simple JSON). When the model edits jax_optimizer.py, retrieve only the rules tagged ["jax", "tmm", "outer_diameter"]. This keeps the relevant rules in the context window.

Example rule format:

{
  "id": "rule-042",
  "files": ["jax_optimizer.py", "two_phase_optimizer.py"],
  "function": "tmm_instrument_from_radii",
  "check": "outer_diameter_mm must come from cfg['outer_diameter'], never default 22.0",
  "severity": "P0",
  "rationale": "22mm is wrong for bass instruments; flange correction shifts pitch"
}

2. The “Two Computers” Coordination Problem

You have two agents on two machines. They are probably: - Both using the same global governance doc - Both editing the same files - Neither knowing what the other changed

This creates semantic merge conflicts that no git merge can detect. Example: - Computer A fixes outer_diameter in jax_optimizer.py - Computer B “optimizes” eval_all and reintroduces the hard-coded 22.0 because it doesn’t see A’s change

What to do instead:

A. Use a “Lock File” for agent sessions Before an agent starts editing, it writes a lock:

{
  "agent": "big-pickle-02",
  "file": "backend/jax_optimizer.py",
  "rules_being_addressed": ["rule-042", "rule-056"],
  "timestamp": "2026-08-05T18:00:00Z"
}

The other agent refuses to edit that file until the lock is released with a validation report.

B. Use “Differential Prompting” Don’t give the agent the full file. Give it: 1. The git diff since last agent session 2. The specific function to edit 3. The constraint rules that apply to that function

This forces the model to reason about changes, not rewrite from scratch.

3. The “Benchmark Blindness” Problem

Your agents optimize for benchmark numbers, not correctness. The benchmark_all.py table shows 0.53c RMS — but that number is computed with wrong outer diameters and wrong unpacking. The agent sees “0.53c” and thinks “good enough, move on.”

What to do instead:

A. Add “Sanity Checks” to the benchmark pipeline Before printing RMS, verify:

assert all(od > id_mm for od, id_mm in zip(outer_diameters, inner_diameters)), \
    "Impossible: outer < inner"
assert len(hp) == n_holes, f"Unpacking bug: expected {n_holes} holes, got {len(hp)}"
assert all(math.isfinite(f) for f in freqs), "TMM returned NaN/inf"

If a sanity check fails, the benchmark aborts with a red error. The agent cannot ignore it.

B. Use “Adversarial Validation” Have a second agent (or a script) whose job is to break the first agent’s code: - Randomize instrument configs (swap flute/clarinet outer diameters) - Check that closed_top affects the result - Verify that changing bore_length by 10mm changes the fundamental by ~15 cents

If the adversary finds an invariant violation, the original agent must fix it.

4. Recommended Governance Doc Restructure

Replace your current governance doc with this hierarchy:

/governance/
  00_architecture.md      # High-level design (read once, not per-prompt)
  01_invariants.md        # MUST NOT violate (checked by CI)
  02_checklists.md        # Per-file constraint checklists
  03_rationale.md         # Why each rule exists (for debugging)
  04_agent_protocol.md    # How agents coordinate (lock files, etc.)

01_invariants.md example:

## Physics Invariants
1. `outer_diameter > inner_diameter` always
2. `bore_length > max(hole_position)` always
3. `closed_top` must match instrument family (clarinet=True, flute=False)

## Code Invariants
4. No hard-coded 22.0 in `tmm_instrument_from_radii` calls
5. No `sorted()` inside differentiable cost functions
6. `n_bore_ctrl` must respect caller parameter

These invariants are checked by a pre-commit script, not by the agent.

5. The “Human in the Loop” Minimum

Even with perfect governance, one critical bug per 500 lines is the current state of the art for AI coding. For your project:

| File | Lines | Critical Bugs Found |
| --- | --- | --- |
| bore_optimizer.py | ~150 | 3 |
| jax_optimizer.py | ~400 | 4 |
| benchmark_all.py | ~350 | 3 |
| two_phase_optimizer.py | ~300 | 5 |
| chromatic_flute.py | ~250 | 3 |

That’s roughly 1 critical bug per 60 lines. No governance doc can fix this alone because the bugs are domain-specific (acoustics) and cross-file (a bug in benchmark_all.py unpacking is invisible to an agent editing jax_optimizer.py).

Minimum viable human review: - Every P0 bug I found could be caught by a 5-minute human review of the diff - Focus human time on: (1) hard-coded constants, (2) tuple unpacking, (3) coordinate system conversions - Use AI for: refactoring, docstrings, test generation, boilerplate

Bottom Line

Your governance docs are probably fine for human developers. The problem is that LLMs are not humans — they are stochastic parrots with goldfish memories. The fix is not “better docs” but mechanical constraints: invariant checks, lock files, differential prompts, and adversarial validation. And yes — a human needs to review every diff for hard-coded constants and tuple unpacking, because those are the two bug classes that AI agents will keep reintroducing forever.